Appendix В 
Information on machine troubleshooting

В.1 General requirements

If the machine cannot be switched on (indicators are not lighted up) when  knob 2 is set  in accordance with  figure С.1 to any division, the rated voltage 220-230 V availability shall be checked in AC electrical mains. Then, by switching the machine off the mains two screws 8 shall be turned out which fix cover 17 or 17A in accordance with figure 1, the cover shall be displaced backwards and taken off. Quality of connection of wires in a yellow 3-position block shall be checked at the control module. The mains cord and the filter of radio interferences shall consecutively be replaced. If the machine cannot be switched on the control module shall be replaced (dismantled mains cord in good order, the filter and the interface module shall be reinstalled in the reversed order).

Machine defecting shall be carried out in accordance with the self-diagnostics program.

Quality of the electric wiring and electrical connections should also be checked in the process of self-diagnostics (wearing, twisting, ruptures shall be excluded).

Electric connection quality shall be checked in the following way:

а) The malfunctioned  electric component shall be found (electric pump, electric valve, electronic module, etc.). For this purpose the self-diagnostics program shall be launched and the result shall be evaluated in accordance with table С.1.

b) The wires which connect the malfunctioned electric component with the electronic module and other electrical components shall be determined in accordance with attachments А, В.

c) In turn, by disconnecting these wires the connection quality shall be checked – the place of electric contact shall be clean, without darkening and traces of burning.

d) a gap in the socket section shall be as follows:

1) for the 3-position yellow plug-and-socket connector at the control module 0.5 mm, not exceeding;

2) for 9-position and 10-position connectors at the control module 1.0 mm, not exceeding;

3) for the plug-and-socket connector at a motor and at a temperature sensor 0.5 mm, not exceeding;

4) for the plug-and-socket connector at a filter of radio interferences 0.5 mm, not exceeding;

5) for rest clamps – 0.65 mm, not exceeding.

If the electric contact quality is unsatisfactory, braid 21 should be replaced in accordance with figure 1.

B.2 Information on the self-diagnostics program

B.2.1 Before start of the self-diagnostics program one should make sure that the machine has been  switched off, and for this purpose key 1 shall be pressed in accordance with figure B.1 (if necessary).

1 – key, 2 – knob, 3 – push button.

Figure B.1 – Control panel

B.2.2 Push button 3 shall be pressed in accordance with figure B.1 and by keeping it pressed key 1 shall simultaneously be pressed in accordance with figure B.1 – the machine will be switched on.

B.2.3 By keeping push button 3 pressed in accordance with figure B.1 knob 2 shall be turned clockwise in accordance with figure B.1 up to a sound signal emergence. Then (during 2 seconds, not exceeding), knob 2 shall be turned counter-clockwise by simultaneously releasing push button 3; knob 2 shall still be turned further in accordance with figure B.1 up to a sound signal emergence. Then the machine self-diagnostics program will be switched on. All indication on the display and a light conductor in the push button will be blinking in the process of program execution.

B.3 For making a pause knob 2 should be turned clockwise in accordance with figure B.1. For continuation of the self-diagnostics program knob 2 should be turned counter-clockwise in accordance with figure B.1.

B.4 For cessation of self-diagnostics  fulfillment key 1 should be pressed in accordance with figure B.1.

B.5 If a malfunction is detected, its code is lighted up on the machine display, and program fulfillment is  terminated.

Codes of main malfunctions, which can be revealed in the self-diagnostics process are shown in table B.1.

Table B.1

| Defect revealing stage | Malfunction code | Possible malfunction | Troubleshooting |
| --- | --- | --- | --- |
| Check of the electric lock of door blocking | “Close the door” | The electric lock of door blocking has wrongly been installed | The electric lock of door blocking (C.1) shall be replaced or its position shall be adjusted |

Table C.1 cont.

| Defect revealing stage | Malfunction code | Possible malfunction | Troubleshooting |  |
| --- | --- | --- | --- | --- |
| Check of the electric lock of door blocking | “Close the door” | The door is not properly closed | The door shall be closed to hear a perceptible click of the lock |  |
|  |  | No electric contact at the electric lock of door blocking | Correct connection of wires of the plug-and-socket connection of the electric lock of door blocking and the control module |  |
|  |  | No electric contact between the control module and the electric lock of door blocking (breakage of a braid wire) | A multimeter shall be used for check of availability of the electric circuit between the control module and the electric lock of door blocking. In case of breakage the braid shall be replaced. |  |
|  |  | Malfunction of the control module | The control module shall be replaced (C.2) |  |
| Check of the electric lock of door blocking | F10 | Malfunction of the control module | The control module shall be replaced (C.2) |  |
| Check of electric valve “1” (switching to 5 sec, water shall be supplied via the “main washing” section”) | - | Malfunction of electric valve “1” and the control module | The electric valve (C3) or the control module (C.2) shall be replaced |  |
| Check of electric valve “2” (switching to 5 sec, water shall be supplied via the “preliminary washing” section”) | - | Malfunction of electric valve “2” and the control module | The electric valve (C3) or the control module (C.2) shall be replaced |  |
| Check of electric valves “1” and “2” (switching on to reach the level “1” by a relay sensor) | F5 | Malfunction of electric valves “1” and “2”, or the sensor relay or the control module | Correct connection of a hose to a water line shall be checked. If water is delivered from a cock, electric valve (C.3) or a level relay sensor C.4), or control module (C.2) shall be replaced |  |
| Check of electric valves “1” and “2” (switching on to reach the level “1” by a relay sensor) | F5 | The filling hose is clogged |  | The filling hose is to be disconnected from the machine. Water passage shall be checked via a filling hose by opening a water supply cock. In case of no availability of water passage the filling hose shall be replaced. |

Table B.1 cont.

| Defect revealing stage | Malfunction code | Possible malfunction | Troubleshooting |
| --- | --- | --- | --- |
| Check of electric valves “1” and “2” (switching on to reach the level “1” by a relay sensor) | F5 | No electric contact between the control module and the electric lock of door blocking (breakage of a braid wire) | A multimeter shall be used for check of availability of the electric circuit between the control module and the electric lock of door blocking. In case of breakage the braid shall be replaced. |
| Check of the motor (motor rotation to the right side with the speed 55 rev/min during 5 seconds. Then, there follows a pause during 2 seconds. (motor rotation to the left side with the speed 55 rev/min during 5 seconds) | F6, or F9, or F12 | Malfunction of the motor or the control module | The motor (C.5) or the control module (C.2) shall be replaced |
| Check of the temperature sensor | F2 (flashing after completion of the self-diagnostics program) | Malfunction of the temperature sensor | The temperature sensor along with the electric heater (C.6) shall be replaced |
| Check of the heater | - | Malfunction of the electric heater or the control module | The temperature sensor along with the electric heater (C.6) or the control module (C.2) shall be replaced |
| Check of the electric pump (electric pump switching on to reach “0” level) | F4 | Malfunction of the electric pump or the control module | Odd objects shall be removed from the electric pump filter in accordance with the requirements of “the Operation Manual” – the flywheel shall freely rotate. Otherwise, the electric pump (C.7) or the control module (C.2) shall be replaced |
| Check of the motor (increase of rotation revolutions by the maximum value, then their decrease to stoppage of the motor) | F6, or F9, or F12 | Malfunction of the electric motor or the control module | The motor (D.5) or the control module (D.2) shall be replaced |

Additionally, malfunctions shown in table B.2 may be detected in the process of self-diagnostics.

Table B.2

| Possible malfunction | Malfunction code | Troubleshooting |
| --- | --- | --- |
| Current frequency in the mains does not comply with the required value | F7 | The machine should be operated in accordance with “the Operation Manual. Current frequency shall be equal to (50±1)Hz |
| Water amount in the tank exceeds the allowed value. Malfunction of the electric valve or the level relay sensor or the control module | F8 | The electric valve (C.3), or the level relay sensor (C.4), or the control module (C.2) shall be replaced |
| Malfunction of the control module or the interface module | F13 or F14 | The control module (C.2) or the interface module (C.8) shall be replaced |

The self-diagnostics program consecutively undergoes all stages of defects revealing, and if no defects were detected in electric components of the machine the program is ended. Three signals are sent by a buzzer. The machine returns to the original position.

Defects shown in table B.3 may be detected in the process of machine operation.

Table B.3

| Possible malfunction | Possible reason of the malfunction | Troubleshooting |
| --- | --- | --- |
| Water is boiling in the tank | Malfunction of the control module | The control module (C.2) shall be replaced |
|  | Malfunction of the temperature sensor | The temperature sensor along with the electric heater (C.6) shall be replaced |
| Water leakage from the tank (in machines СМА 45Y144-А-ХХХ, СМА 50С124-А-ХХХ, СМА 50С144-А-ХХХ the code of malfunction F15 begins to flash) | The filter handle in the electric pump is not turned in against the rest | The filter handle shall be turned in against the rest |
|  | Poor connection or rupture in the branch, tube, discharge sleeve, discharge or filling hose | Availability of clamps and quality of connections in branch, tube, discharge sleeve, discharge or filling hose shall be checked. If necessary, they shall be replaced (C.9) |
|  | Water leakage through the door sealant or the sealant | Tightness shall be restored or the door sealant (C.10) or the sealant (C.11) shall be replaced |
|  | A crack in the tank cover | The tank cover shall be replaced |
| Odd knocks, vibration and clicks during drum rotation | Pulley fastening is loose | Rear panel 6 shall be dismantled in accordance with figure 1. Screw 3 shall be tightened in accordance with figure 4. Screw tightening force shall be equal to (35±1) N·m |
|  | Wear of the belt (cracks, chafes, cord fibers are visible on its surface) | The belt (C.5) shall be replaced |
| Odd knocks, vibration and clicks during drum rotation | Fastening of counterweights is loose | Cover 17 or 17A shall be dismantled in accordance with figure 1, screws of fastening the counterweights shall be tightened. Screw tightening force shall constitute from 14 to 17 N·m |

Table В.3 contd.

| Possible malfunction | Possible reason of the malfunction | Troubleshooting |
| --- | --- | --- |
| No rotation of the drum. The motor operates | Belt rupture | The belt (C.5) shall be replaced |
|  | The belt has run off the pulley | The belt (C.5) shall be mounted |
|  | Pulley fastening is disrupted | Rear panel 6 shall be dismantled in accordance with figure 1. Screw 3 shall be tightened in accordance with figure 4. Screw tightening force shall be equal to (35±1) N·m |
| No water drainage | Malfunction of the electric pump or the control module | Odd objects shall be removed from the electric pump filter in accordance with the requirements of “the Operation Manual” – the flywheel shall freely rotate. Otherwise, the electric pump (C.7) or the control module (C.2) shall be replaced |
| Water cannot be heated up. Malfunction code F3 is flashing | Malfunction of the electric heater or the control module | The electric heater shall be replaced along with the temperature sensor (C.6) or the control module (C.2). Nut tightening force shall be equal to (4±0.5) N m |
| The machine does not operate | “Protection from children” function is on | “Protection from children” function shall be switched off in accordance with “the Operation Manual” |
| The machine does not operate, display indication areas are not flashing | No voltage in the electrical mains | Voltage in the electric socket shall be checked with a multimeter |
|  | Malfunction of the mains plug | The mains cord shall be replaced |
|  | No electrical contact between the control module and the filter of radio interferences | A multimeter shall be used for checking the electrical circuit availability between the control module and the filter of radio interferences. In case of circuit breakage the braid shall be replaced |
| The washing program cannot be fulfilled. The indicator of operation is flashing, START push button is lighted up | Poor contact in braid plug-and-socket connectors | Contact shall be provided in braid plug-and-socket connectors |
|  | Malfunction of the control module | The control module shall be replaced (C.2) |
| The door cannot be opened after completion of the washing program after 3 minutes | The machine door is not centered, a door hook travel is insufficient | The door shall be centered, door hook travel shall be checked and adjusted |
| Water drainage is too slow (more than 1 minute) | The drainage hose is wrongly installed | The drainage hose shall be installed in accordance with “the Operation Manual” |
|  | The filter of the drainage electric pump is clogged | The filter of the electric pump shall be cleaned in accordance with “the Operation Manual” |
|  | A detergent is used which is not intended for automatic washing machines | The filter of the electric pump shall be cleaned in accordance with “the Operation Manual” |

Table B.3 cont.

| Possible malfunction | Possible reason of the malfunction | Troubleshooting |
| --- | --- | --- |
| No water drainage | Symbol (pressing is off) is set by means of a push button of selection of pressing revolutions | The push button of selection of pressing revolutions shall be used in accordance with the fabric type |
|  | Poor contact in joints of the electric braid | Contact shall be provided in braid joints at the electric pump and the control module |
|  | Malfunction of the drainage electric pump | The electric pump (C.7) shall be replaced |
|  | Malfunction of the control module | The control module (C.2) shall be replaced |
| A signal is constantly heard during the washing process | Malfunction of the interface module | The interface module (C.8) shall be replaced |
|  | Malfunction of the control module | The control module (C.2) shall be replaced |
|  | Poor contact of the cable with modules | Contact shall be provided |
| After program completion the washing process is resumed | Malfunction of the control module | The control module (C.2) shall be replaced |
| Programs are spontaneously switched over | Malfunction of the control module | The control module (C.2) shall be replaced |
|  | Malfunction of the interface module | The interface module (C.8) shall be replaced |
| Damage or fall-out of the ridge | Mechanical damage of the ridge | Depending on the damage character the following shall be done: the damaged ridge shall be reinstalled to its place being turned by 180º or a new ridge shall be mounted instead of the damaged one. For this purpose a hook shall be bent in the ridge center |